Safe Job Procedures

| Name of Job: Band saw | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Exposed blades - cuts, amputations | Electrical shock | Excessive noise | Saw dust in eyes |
| --- | --- | --- | --- |
| Inhalation of saw dust particles |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Safety glasses | Dust mask | Steel toed boots | Hearing protection |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Ensure saw is unplugged or locked out

- Inspect work area to ensure clean

- Inspect electrical cords, switches, blade and guards for defects (all defective items will be tagged out of service)

- Inspect wood to ensure there are no defects or nails/screws in wood

- Place wood on work platform and measure height of wood

- Adjust guard to 1/8 inch above the height of wood

- Turn on saw and inspect blade as it runs to ensure no defects

- Push wood through saw and wait until blade stops to remove wood from platform

- Use a push stick if wood is small

- Unplug or lock

- If there is a lot of saw dust, wear a dust mask while cleaning