Safe Job Procedures

| Name of Job: Boosting Batteries | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Sulfuric acid | Explosive hydrogen gas | Awkward location |  |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Acid resistant gloves | Eye protection |  |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow? .

- Check cables for wear, frays, cracks and/or loose clamps

- Ensure ends of cables do not touch when attached to batteries as sparks may occur

- Connect booster cables to the uncharged battery first to prevent sparks

- Connect the cables by attaching positive to positive and negative to a metal bracket or mount directly on the engine of the vehicle requiring boost

- Make last connection to the battery of the vehicle giving charge

- Remove cables in reverse order once uncharged vehicle is started being careful not to let the ends touch as arcing may occur