Safe Job Procedures

| Name of Job: CAT Operator | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Other workers and equipment | Pinch points | Slips, trips & falls | Equipment malfunction |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Hard hat | Safety glasses | Safety vest |
| --- | --- | --- | --- |
| Gloves |  |  |  |

What is a proper safe job procedure to follow?

- Use Cold Start Procedure and do pre-trip inspection while machine is warming up

  - Check for:

    - Hydraulic or fuel leaks or loose bolts

    - Lights and back up alarm are working

    - Hydraulics are running properly

- Mount and dismount machine using 3 point contact

- Proceed to work area checking steering and brakes as you go

- Maintain proper distance from other workers and equipment and always look directly behind before backing

- Pay constant attention to all gauges

- When stopped, check machine over and do not drag winch cable or have hook dangling loosely

- Park in safe area and drop the hydraulics

- Use Hot Shut Down Procedure

- Check over machine, clean radiator and tracks when finished.