Safe Job Procedure for Circular Saw Use

- Pre-operation check/inspection

- Wear appropriate personal protective equipment.

- Disconnect the power cord from the electrical outlet before inspecting, adjusting, cleaning or repairing the saw or changing blades.

- Inspect the saw and power cord for damage prior to each use.

- Ensure the blade is sharp.

- Ensure the blade is the correct type for the material to be cut.

- Ensure the blade is securely mounted.

- Ensure the blade is in good condition; no nicks or cracks.

- Ensure the retracting lower blade guard is working freely.

- Check the blade for proper blade rotation.

- Check the material for any defects such as warps or knots and foreign objects such as nails, staples or screws.

- Adjust the blade depth to no more than ½ inch (1.3 cm) beyond the lower face of the material.

- Operation

- Secure and adequately support the material to be cut.

- To rip material use a guide that is clamped or nailed to the material.

- Be aware of sawdust and debris from cutting the material.

- Keep all electrical cords clear of the cutting area.

- Ensure an adequate length of cord is available to fully complete the cut.

- Use both hands to operate the saw; one on the trigger handle and the other on the front knob.

- Keep your body to the left side of the blade; never in line with the saw blade just in case of a kickback.

- The weight of the saw must always be on the clamped side of the stock.

- Start the saw with the shoe in contact with the stock, but the blade clear of the stock.

- Allow the saw to attain full power before cutting.

- Always cut in a straight line.

- Allow off-cuts to fall.

- If the blade binds or when interrupting a cut, release the trigger and hold the saw motionless until the blade comes to a complete stop.

- Keep the guard free from an accumulation of sawdust and chips.

- Ensure the retracting lower blade guard is fully returned and the blade has fully stopped before setting the saw down.

- Disconnect the power cord before adjusting or changing the blade or performing regular maintenance.