Safe Job Procedures

| Name of Job: Cutting Torch | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Burns | Eye injuries | Compressed gas | Fire/Explosion |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Eye protection | Fire extinguisher | Face shield | Hand protection |
| --- | --- | --- | --- |
| Helmet | Fire retardant coveralls |  |  |

What is a proper safe job procedure to follow?

- Ensure that bottles are secured in the upright position

- Zero the gauges

- Mount the gauges to the bottles – red hose to Acetylene – green hose to Oxygen

- Open valves

- Set gauges by turning regulator dial clockwise – 5lb Acetylene – 40lbs Oxygen

- Turn on acetylene – ¼ turn at torch handle

- Light with striker

- Increase the acetylene until the black smoke decreases

- Turn on the oxygen at the torch handle

- Increase the oxygen until the desired flame is achieved (blue short tip with no feathers)

- Cut as required

- Shut off in the reverse order