Safe Job Procedures

| Name of Job: Extinguishing & Dismantling Oxy Acetylene Equipment | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Burns | Inhalation of toxins | Fire and explosion hazard |  |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Hand protection | Eye protection |  |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Close Oxygen needle valve

- Close acetylene valve at torch end

- Close acetylene cylinder valve

- Close oxygen cylinder valve

- Drain acetylene by opening acetylene torch valve for a moment, then close again

- Drain oxygen in same manner as acetylene

- Re-open both torch valves

- Release pressure adjusting crews on both regulators

- Regulators and torches cannot be disconnected