Safe Job Procedures

| Name of Job: Lighting Oxy Acetylene Torch | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Inhalation of smoke | Burns | Fire/explosion | Inhalation of toxins/chemicals |
| --- | --- | --- | --- |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Eye protection | Hand protection |  |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Purge the line of any air or explosive mixture by opening the torch needle valves one at a time

- Open acetylene needle valve about a half turn

- Do not open oxygen needle valve yet

- Using striker, ignite the acetylene ensuring the flame will not cause physical or property damage

- Open oxygen needle valve and balance mixtures to the desired combination