Safe Job Procedures

| Name of Job: Supporting (Chocking) Dump Boxes | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Serious injury | Pinch points | Potential death |  |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Eye protection | Steel toed boots | Hand protection | Hard hat |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Park and block truck in neutral with the power take off (PTO) and pump engaged

- Raise truck box to desired level

- A Stiff Leg is a permanently attached pivot and rest, and is the preferred method of support

- If not available, use the following:

  - 6”x 6”x 6” long timber block across the truck frame between the box and subframe as close as possible to the box hinge

  - 2 pieces of 4”x 4”x 6” timber placed vertically between tandem tire and the box

  - Pin locks provided by the manufacturer to secure the box in the elevated position

- Only keep the box lifted for as long as required