Safe Job Procedures

| Name of Job: Tree Falling with Chain Saw | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Amputation | Equipment malfunction | Pinch Points | Slips, trips and falls |
| --- | --- | --- | --- |
| Overhead wires | Noise |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Hard hat | Face shield | Communication device |
| --- | --- | --- | --- |
| Chainsaw chaps | Hand protection | Safety vest | Hearing protection |

What is a proper safe job procedure to follow?

- Wear all appropriate PPE at all times

- Do a pre job hazard assessment

- Pick a tree – look for hazards

- Brush out tree and plan an escape route

- Make correct undercut – correct Dutchman if necessary

- Make side cuts if necessary

- Start back cut

- Start the wedge

- Complete the back cut

- Drive the wedge

- Use escape route – Do not turn your back on a falling tree

- Wait 15 seconds or longer for the tree and branches to fall