Safe Job Procedures

| Name of Job: Vehicle Daily Maintenance | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Equipment failure | Collision | Pinch points |  |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Eye protection | Hand protection |  |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Walk around machine to check for damage, vandalism or any loose or worn parts.

- Check engine for:

  - Any frayed or damaged belts

  - Any damaged or leaking hoses

  - Engine mounting bolts are in place, tight and not damaged

- Check all fluid levels (some fluids may require engine to be on, check manufacturers recommendations)

- Get into vehicle, using 3 point contact. Check for fire extinguisher and first aid kit.

- Turn on engine.

- Check all gauges. Ensure that:

  - Oil pressure is normal

  - Temperature is normal

  - Hydraulic pressure is normal (if applicable)

- Put machine in reverse to ensure back up alarm is working

- Do another walk around to check all lights, get assistance if required

- Check brakes are working when pulling away to drive

- Proceed to job