Safe Job Procedure for Step Ladders

- Selection and inspection

- Wear appropriate personal protective equipment.

- Use a ladder that is designed for the work task. Check the ladder’s strength/grade, type/use and CSA approval.

- Select a stepladder that is about 1 m (3 ft) shorter than the highest point you have to reach. This gives a wider, more stable base and places the shelf at a convenient working height.

- Use a fibreglass ladder when doing electrical work or when working in close proximity to electrical equipment.

- Inspect the stepladder before each use for the following;

- Damaged or worn non-slip feet.

- Loose nails, screws, bolts or nuts.

- Loose or faulty spreaders, locks and other metal parts in poor condition.

- Rotten, decayed or warped rails on wooden ladders.

- Cracks and exposed fibreglass on fibreglass ladders.

- Cracked, split, worn or broken rails, braces, steps or rungs.

- Sharp edges on rails and rungs.

- Rough or splintered surfaces.

- Corrosion, rust, oxidization and excessive wear, especially on treads.

- Twisted or distorted rails. Check by sighting along the rails.

- Missing identification labels.

- Loose or bent hinges and hinge spreaders.

- Broken stop on a hinge spreader.

- Do not use a broken or unsafe ladder, attach a warning tag, take it out-of-service and advise your supervisor.

- Operation

- Get help when handling a heavy and/or long stepladder.

- Check for overhead electrical wires or other hazards before setting up the ladder.

- Clear the area around the base of the ladder of debris and other objects.

- Set up barricades and warning signs wherever necessary.

- Open the stepladder spreaders and shelf fully; lock into correct position.

- Check stability. Ensure that all ladder feet are on a firm, level and non-slippery surface.

- Place the stepladder at a right angle to the work with the back of the steps facing the work.

- Keep the stepladder close to the work.

- If your boots are muddy then clean the soles before climbing the ladder.

- Avoid climbing with wet soles. Ensure your footwear is in good condition.

- Face the stepladder when climbing up and down. Keep your body centered between the side rails. Do not work from either the top two steps of a stepladder.

- Maintain a firm/secure grip. Use both hands when climbing.

- Maintain a three point contact by keeping two hands and one foot, or two feet and one hand on the ladder at all times.

- Ensure all items are secure on the stepladder.

- Storage and maintenance

- Clean a fibreglass ladder every three months. Spray it lightly with a clear lacquer or paste wax.

- Protect a wooden ladder with a clear sealer or wood preservative.

- Return ladders to the designated storage area after use.

- Store ladders where they are protected from the weather.

- Keep ladders clean and free of foreign materials.

- Keep wooden ladders in a well-ventilated location, away from dampness and excessive heat.