Sample General Safe Work Practice for Chop Saw

For operation of the chop saw, face shield and safety glasses must be worn.

- Loose clothing, jewelry, or any dangling objects including long hair should not be worn as they may catch in the rotating parts of the saw. Hair must be contained.

- The maximum cut for the machine must not be exceeded.

- Ensure all adjustments are secure before making a cut.

- Use clamps to secure and support the work piece to a stable platform. Do not use a length stop on the free scrap end of a clamped work piece

- Do not cut more than one work piece at a time.

- Before turning on the saw, perform a dry run of the cutting operation to ensure that no problems will occur when the cut is made.

- All guards must be in place and operating. If a guard seems slow to return to its normal position or hangs up, adjust it or repair it immediately. Unplug before adjusting or cleaning.

- Do not start the saw with the blade touching the work piece. Allow the blade to reach full speed before contacting the work piece.

- Avoid reaching over the saw line.

- Do not cross arms when cutting.

- Hands and fingers must be kept clear of the path in which the blade travels.

- When using the right hand to pull the saw down, keep the left hand, especially the thumb well clear of the line of cut.

- Clean the lower guar frequently to help visibility and movement. Unplug before adjusting and cleaning

- Use only the recommended RPM and sizes of blades.

- Regularly check and tighten the blade, be sure to lockout or unplug equipment

- When installing or changing a blade, ensure that the blade and its related washers and fasteners are correctly positioned and secured on the saws arbor.

- Long material should be supported at the same height as the saw table. 18. Use the brake if one is installed. To avoid contact with a coasting blade, do not reach into the cutting area until the blade has come to a complete stop.

- After completing a cut, release the trigger switch and allow the blade to come to a complete stop, and then raise the blade from the work piece. If the blade stays in the cutting area after the cutting is complete, injury can result from accidental contact.