Safe Work Practice

| Name of Job: Use of Portable Grinders | Development Date: February 17th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Gloves, loose clothing or jewelry being caught | Flying debris | Sparks | Electrocution |
| --- | --- | --- | --- |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Face shield | Safety glasses | Hand protection |
| --- | --- | --- | --- |

What are some good general safe work practices?

- DO familiarize yourself with the grinder operation before commencing work.

- DO ensure proper guards are in place and that safety glasses, face shields, gloves and safety boots are worn when using portable grinders.

- DO NOT exceed the maximum wheel speed (every wheel is marked).  Check the speed marked on the wheel and compares it to the speed on the grinder.

- DO check the wheels before mounting them, for cracks and defects, ensure that the mounting flanges are clean and the mounting blotters are used.  Do not over tighten the mounting nut.

- DO run newly mounted wheels at operating speed to check for vibrations before grinding.

- DO NOT use grinders near flammable materials.

- DO NOT use the grinder for jobs for which it is not designed, such as cutting.