Safe Work Practice

| Name of Job: Power and Hand Tool Use | Development Date: March 29th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Risk of eye injury from flying particles. | Use of various machinery and tools | Electric shock | Working with hand tools, powered tools and heavy powered equipment |
| --- | --- | --- | --- |
| Repetitive motion strain |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Hearing Protection | Eye Protection | Foot protection | Fall Arrest Equipment |
| --- | --- | --- | --- |
| Respirator Equipment | Skin Protection | Hearing Protection |  |

What are some good general safe work practices?

- DO ensure Electrical tools have 3 wire (grounding) cord and plug, excluding double insulated tools.

- DO ensure that grinder discs, buffers and stones to be used only for designed application and at rated speed.

- DO ensure that stationary grinders have properly adjusted tool rests and stones to be properly dressed.

- DO ensure that angle grinders have Original Equipment Manufacturer (O.E.M.) guard.

- DO make sure On/off switches are functional and positioned so Operator has access.

- DO NOT use accessories unless they are designed for use with the tools specified.

- DO ensue that saw blades are designed for the product being cut and at the rated speed, O.E.M. guards must be in place and functional.

- DO ensure that Chisels, punches, hammer, wrenches, etc. to have all burrs ground from striking area.

- DO ensure that Chisels, punches, screwdrivers, etc. to have tips properly dressed.

- DO ensure that Cracked a/o splintered handles are replaced.

- DO ensure all tools are cleaned after use and repairs made before being properly stored.

- DO ensure that Tools are used for designed purpose only.

- DO ensure that tool repairs are performed by qualified personnel, using O.E.M. parts or equivalent.

- DO follow tool safe work procedures step by step.