Sample General Safe Work Practice for Propane Torch

A flame from a propane torch can reach temperatures of over 1093˚c. Workers applying torch on products can receive serious burns from both the torch flame and the hot modified bitumen sheets they are applying.

- When using a torch, workers must wear additional protective clothing (gloves, eye protection).

- Prior to use, ensure that torching equipment is in good working order and the cylinder valves are clean. Check that fittings, hoses and heads are secure.

- DO NOT USE defective equipment.

- Use soapy water to check connections for leaks.

- Only use a spark lighter or electronic starter to light torch.

- Protect the propane hose from damage by:

  - Keeping torch flame away from hose.

  - Keeping hose free of kinks.

  - Not running over hose with equipment.

  - Not using the hose to lift the cylinder.

- A torch flame is difficult to see in daylight, be aware of and keep away from the flame. 8. NEVER LEAVE AN OPERATING TORCH UNATTENDED.

- Other than the operator, all workers should stay at least 1 metre away from the torch.

- Set torch units into support leg position when not in use.

- To shut off torch, close cylinder valve first, let gas burn out, close torch valve.

- At the end of the day, disconnect hoses and store properly.