Safe Work Practice

Using Bench Grinders

Severe injury may occur if appropriate personal protective equipment is not used and maintained.

- Always protect your eyes with goggles or a face shield when grinding.

- When using bench grinders, check the tool rest for the correct distance from the abrasive wheel, maximum 1/8” or 3 mm.

- Replace the grindstone when adjustment of the rest cannot provide 1/8” or 3 mm clearance.

- If the wheel has been ground to an angle or is grooved, reface the wheel with the appropriate surfacing tool or replace the wheel.

- Each time a grinding wheel is replaced, check the maximum approved speed (stamped on the wheel bladder) against the shaft rotation speed of the machine to ensure the safe speed is not exceeded.

- A grinding wheel must not be operated at speeds exceeding the manufacturer's recommendation.

- The flanges supporting the grinding wheel should be a maximum of 1/3 the diameter of the wheel and must fit the shaft rotating speed according to the manufacturer’s recommendation.

- Bench grinders are designed for peripheral grinding. Do not grind on the side of the wheel.

- Do not stand directly in front of the grinding wheel when it is first started.

- Wear CSA-approved hearing protection.

For further information, see the appropriate current Occupational Health & Safety Legislation.