Safe Work Practice

Portable Grinders

Abrasive wheels can cause severe injury. Proper storage, use and maintenance of wheels must be observed.

- Familiarize yourself with the grinder operation before commencing work.

- Ensure proper guards are in place.

- Never exceed the maximum wheel speed RPM (every wheel is marked).

- Check the speed marked on the wheel and compare it to the speed on the grinder.

- When installing the wheel, check for cracks and defects. Ensure mounting flanges are clean and the mounting blotters are used. Do not over tighten the mounting nut.

- Before grinding, run the newly mounted wheel at operating speed, checking for vibration.

- Do not use grinders near flammable materials.

- Never use the grinder for jobs it is not designed for, such as cutting.

- Wear CSA-approved personal protective equipment including eye, face, hand, foot, and hearing protection.

For further information, see the appropriate current Occupational Health & Safety Legislation.