|  | 35 MacDonald Ave, Dartmouth, NS, B3B 1C6 Tel: 902 468 6696 Toll Free NS: 1 800 971 3888 Web: www.constructionsafetyns.ca |
| --- | --- |

SAMPLE GENERAL SAFE WORK PRACTICE FOR MANUAL MATERIALS HANDLING

Potential Health and Safety Hazards

- Physical hazards; lighting.

- Ergonomic hazards; force (lifting and pushing/pulling) and posture (bending, reaching and twisting).

- Energy hazards; gravity (falling objects).

- Materials handling hazards; lifting, pushing/pulling and hazardous substances.

- Work practice hazards; following established safe work practices and procedures, and general housekeeping practices.

Safe Work Practices

- Wear appropriate personal protective equipment such as work gloves when handling objects with sharp edges and safety footwear when handling heavy objects.

- Ensure there is adequate clearance for safe lifting/material handling.

- Ensure storage areas are kept tidy, well organized and free of clutter.

- Use a hand truck, cart, dolly, wheelbarrow, etc to move heavy, awkward or bulky objects. Ask for assistance.

- Know how to safely handle controlled WHMIS products.

- Reduce repetition as much as possible by pacing your work and by varying tasks.

- Use a stepladder or step stool to reach high places.

- Use a power grasp for loads with handles.

- Use a ledge grasp for loads without handles.

- Wherever possible use an elevator to move supplies and equipment between floors. Ask for assistance and/or use a two-wheeled hand truck dolly to move heavy, awkward or bulky items up or down stairs if an elevator is not available.

- Do not lift or carry items by the packing straps or cords.

- Do not use a box, desk or chair to reach high objects.

- Do not lift a load if you are not sure that you can handle it safely.