|  | 35 MacDonald Ave, Dartmouth, NS, B3B 1C6 Tel: 902 468 6696 Toll Free NS: 1 800 971 3888 Web: www.constructionsafetyns.ca |
| --- | --- |

SAFE WORK PRACTICE

| Name of Job: Welding, Cutting and Burning | Development Date: | Developed By: | Approved By: | Review Date: |
| --- | --- | --- | --- | --- |

Possible Hazards Present

| Burns | Compressed gas | Fire/Explosion | Repetitive Motion /Strain |
| --- | --- | --- | --- |

Personal Protective Equipment (PPE) and Devices Recommended

| Leather wielding clothing | Fire extinguisher | Helmet | Hand protection |
| --- | --- | --- | --- |
| Fire retardant coveralls | Fire retardant Gloves | Foot Protection |  |

What are some general safe work practices?

- DO ensure that adequate ventilation is supplied since hazardous fumes can be created during welding, cutting or burning.

- DO NOT start work without proper authorization.

- DO have fire fighting or prevention equipment on hand before starting welding, cutting or burning.

- DO check the work area and remove any combustible material and possible flammable vapours before starting work.

- DO NOT work alone.  A fire or spark watch should be maintained.

- DO check cables and hoses to protect them from slag or sparks.

- DO NOT weld or cut lines, drums, tanks, etc. that have been in service without making sure that all precautions have been carried out and permits obtained.

- DO NOT enter, weld or cut in a confined space without meeting the requirements of Occupational Safety General Regulations Part 12 - Confined Space Entry.

- DO use fire resistant materials (blankets, tarps) to control or contain slag and sparks.

- DO NOT cut or weld where sparks and cutting slag with fall on cylinders (move all cylinders away to one side).

- DO open all cylinder valves slowly.  The wrench used for opening the cylinder vales should always be kept on the valve spindle when the cylinder is in use.