Safe Work Practice Boosting a Battery

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Awkward/sustained postures - bend, reach, lift |  |
| Forceful exertions - lifting |  |
| Repetitive movements |  |
| Vibration |  |
| Compression |  |
| Sharp points/edges - sharp materials |  |
| Pinch points - bin lids |  |
| Materials falling - bin lids |  |
| Surfaces causing falls - icy, snow in winter |  |
| Moving machinery |  |
| Chemicals |  |
| Biological pathogens - contact with waste |  |
| Electrical |  |
| Extreme heat/cold |  |
| Noise |  |
| Combustibles/flammables |  |
| Risk of falling |  |
| Other |  |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |

Wear High-Vis vests or other types of light and/or reflective clothing (if available) while performing this task in high traffic areas.

c)    Position the donor vehicle engine close to the receiver vehicle battery, out of direct traffic. Do not let the vehicles touch each other.

d)    Clean terminals of both batteries so the “+” and “- “marks can be clearly seen.

e)    Connect the red cable to the “+” posts of each vehicle battery.

f)     Connect the black cable to the “- “post of the good donor battery.

g)    Lastly, connect the other end of the black cable to an unpainted metal part of the vehicle engine. Note: if vehicle(s) are equipped with MRS radio, power supply to radio must be disconnected before boosting.

h)   Start the donor vehicle engine (note that some newer vehicles advise to leave engine off due to internal computer sensing issues-consult owner’s manual).

i)     Start the receiver vehicle engine.

j)     Once the car is running, remove the cables in reverse order.

k)    Allow receiver vehicle engine to run for at least 10 minutes in order to sufficiently recharge the battery.

DON’T:

a)    Do not attempt to auxiliary boost a frozen battery or from which all the electrolyte fluid has evaporated

b)    Do not attempt an auxiliary boost if the donor voltage source is greater than 15 volts (e.g. heavy equipment).

c)    Do not perform this task in the presence of flammable materials.