Safe Work Practice Chipper

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Awkward/sustained postures - bend, reach, lift |  |
| Forceful exertions - lifting |  |
| Repetitive movements |  |
| Vibration |  |
| Compression |  |
| Sharp points/edges - sharp materials |  |
| Pinch points - bin lids |  |
| Materials falling - bin lids |  |
| Surfaces causing falls - icy, snow in winter |  |
| Moving machinery |  |
| Chemicals |  |
| Biological pathogens - contact with waste |  |
| Electrical |  |
| Extreme heat/cold |  |
| Noise |  |
| Combustibles/flammables |  |
| Risk of falling |  |
| Other |  |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |

Note:

- Safety devices and controls, such as emergency shut-off devices, are tested and verified to be functioning properly before the chipper is used.

- Workers are trained in safe work procedures, including operating wood chipper safety devices and safety controls. These procedures should be based on the manufacturer’s recommendations for each machine.

- At least two workers are in close contact with each other when operating the chipper.

- Workers wear close-fitting clothing, gloves without cuffs, trousers without cuffs, and skid-resistant foot wear. Clothing should be kept tucked in.

- Workers’ hands and feet remain outside the infeed hopper.

- Workers feed brush and limbs into the infeed hopper butt end first.

- Workers feeding material are positioned at the side of the machine to allow quick operation of the emergency shut-off device and minimize risk of entanglement in branches. Because of differences among machines, the manufacturer’s operating manual should be consulted for guidance. Safe feeding of some disc-type chippers requires the worker to be on the right side.

- Workers walk away once the feed mechanism has grabbed the material.

- Workers lay short material on top of longer material that’s feeding or use a longer branch to push it through the infeed hopper.

- Workers load small raked-up material such as twigs and leaves directly into the chip truck or in trash cans or bags instead of feeding it into the chipper.

- Workers keep the area around the wood chipper free of slipping and tripping hazards.

Workers wear hard hats, eye protection and hearing protection.