Safe Work Practice Dump Truck

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Awkward/sustained postures - bend, reach, lift |  |
| Forceful exertions - lifting |  |
| Repetitive movements |  |
| Vibration |  |
| Compression |  |
| Sharp points/edges - sharp materials |  |
| Pinch points - bin lids |  |
| Materials falling - bin lids |  |
| Surfaces causing falls - icy, snow in winter |  |
| Moving machinery |  |
| Chemicals |  |
| Biological pathogens - contact with waste |  |
| Electrical |  |
| Extreme heat/cold |  |
| Noise |  |
| Combustibles/flammables |  |
| Risk of falling |  |
| Other |  |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |

Inspections:

- Conduct a documented pre-trip to inspect the truck to ensure it is in safe operating condition.

- Check the dump box lift mechanism regularly to ensure the pump, reservoir, hoses, and other critical components are free of defects and in good working order.

- Check the condition of the brakes and adjust them regularly to ensure they are functioning properly.

- Ensure the required government inspections of the vehicle are conducted and are current.

- Carry all registrations and permits necessary when travelling on public roads.

- Fill in your logbook as required.

- Conduct a documented post trip inspection at the end of your day and report any safety issues to the truck mechanic. Have him sign off your logbook when the repairs are completed.

Operating:

- Wear your seatbelt at all times when your truck is in motion.

- Make sure you have all the required radio frequencies before you start on a new haul.

- Ask for all the hazard information before you start a new haul.

- Call your kilometers according to the protocol posted at the start of the road.

- If in doubt, ask if you are unsure of the calling protocol or if you are on the correct channel.

- Drive with the headlights on at all times.

- Drive within the posted speed limits and/or within safe speeds determined by the conditions of the road.

- Always get into and leave the truck in a safe manner using the handholds provided to prevent slipping and tripping. (Use the 3-point mount/dismount.)

- Stay inside the truck cab or out in front of the unit while being loaded.

- Watch the load being loaded with your mirrors; notify the loader operator right away if you see any issues especially on his blind side.

- Communicate with the loader man when the loading is complete.

- Check your weight. Do not overload for truck, bridge, or road limits.

Unloading

- Approach the dumping area in a safe manner. Watch for other workers and machinery which might be present. Confirm unloading location with spreader/grader/cat/foreman.

- Follow the safe dumping procedures established at the unloading site.

- Wear the required P.P.E. when out of the cab.

- Apply truck parking brake before leaving the cab.