Safe Work Practice Jackhammer

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Awkward/sustained postures - bend, reach, lift |  |
| Forceful exertions - lifting |  |
| Repetitive movements |  |
| Vibration |  |
| Compression |  |
| Sharp points/edges - sharp materials |  |
| Pinch points - bin lids |  |
| Materials falling - bin lids |  |
| Surfaces causing falls - icy, snow in winter |  |
| Moving machinery |  |
| Chemicals |  |
| Biological pathogens - contact with waste |  |
| Electrical |  |
| Extreme heat/cold |  |
| Noise |  |
| Combustibles/flammables |  |
| Risk of falling |  |
| Other |  |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |

Due to high Dba exposure (or noise) when jack hammering all workers must wear hearing protection.

- Inspect air hoses.

- Check hose fittings for tightness and proper inserts are installed.

- Protect hose from being run over.

- Workers assemble work tools, jackhammer, chisels, hoses etc.

- Workers inspect tools for operating condition.

- Workers assemble hose and wire up connections.

- Workers check compressor, start, let warm up, and begin jackhammer function.

- Let the jack hammer do the work for you.

- Do not over stretch.

- Maintain safe distance from surrounding objects (pinch points).

- Be aware of jack hammer placement at all times.

- Store tools appropriately. Read and understand the manual that accompanies your plate tamper.

- When the task is completed, shut down the machine and remove it from the work area. Store the machine properly.