Safe Work Practice Propane Torch

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Awkward/sustained postures - bend, reach, lift |  |
| Forceful exertions - lifting |  |
| Repetitive movements |  |
| Vibration |  |
| Compression |  |
| Sharp points/edges - sharp materials |  |
| Pinch points - bin lids |  |
| Materials falling - bin lids |  |
| Surfaces causing falls - icy, snow in winter |  |
| Moving machinery |  |
| Chemicals |  |
| Biological pathogens - contact with waste |  |
| Electrical |  |
| Extreme heat/cold |  |
| Noise |  |
| Combustibles/flammables |  |
| Risk of falling |  |
| Other |  |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |

A flame from a propane torch can reach temperatures of over 1093˚c. Workers applying torch on products can receive serious burns from both the torch flame and the hot modified bitumen sheets they are applying.

- When using a torch, workers must wear additional protective clothing (gloves, eye protection).

- Prior to use, ensure that the torching equipment is in good working order and the cylinder valves are clean. Check that fittings, hoses and heads are secure.

- DO NOT USE defective equipment.

- Use soapy water to check connections for leaks.

- Only use a spark lighter or electronic starter to light a torch.

- Protect the propane hose from damage by:

  - Keeping torch flame away from hose.

  - Keeping hose free of kinks.

  - Not running over hose with equipment.

  - Not using the hose to lift the cylinder.

- A torch flame is difficult to see in daylight, be aware of and keep away from the flame. 8. NEVER LEAVE AN OPERATING TORCH UNATTENDED.

- Other than the operator, all workers should stay at least 1 meter away from the torch.

- Set torch units into support leg position when not in use.

- To shut off the torch, close cylinder valve first, let gas burn out, close torch valve.

- At the end of the day, disconnect hoses and store properly.