Safe Work Practice Roofing

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Awkward/sustained postures - bend, reach, lift |  |
| Forceful exertions - lifting |  |
| Repetitive movements |  |
| Vibration |  |
| Compression |  |
| Sharp points/edges - sharp materials |  |
| Pinch points - bin lids |  |
| Materials falling - bin lids |  |
| Surfaces causing falls - icy, snow in winter |  |
| Moving machinery |  |
| Chemicals |  |
| Biological pathogens - contact with waste |  |
| Electrical |  |
| Extreme heat/cold |  |
| Noise |  |
| Combustibles/flammables |  |
| Risk of falling |  |
| Other |  |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |

- DO ensure that you check the roof surface for ice or dampness prior to climbing onto the roof

- DO ensure that traffic on a roof should be avoided as ice, snow and frost make the surface hazardous

- DO ensure when removing snow, avoid direct contact with shingles as this will scrape off the protective granules

- DO ensure when applying shingles in temperatures below 0°C, handle shingles with care as they can become brittle.

- DO maintain a safe distance from power lines and ensure neither your body nor any equipment come into contact with the lines. When in doubt, call NS Power to have the lines insulated or de-energized.

- DO wear clothing appropriate to the task. Remember you can get sunburn and hypothermia at the same time when working at heights.

- DO ensure the roof is kept clean of debris such as wood, shingle parts and dirt. Be aware of loose items, such as electrical cords and tools.

- DO NOT walk on the roof when not necessary, as this may cause even more damage and increase your risk of falling.

- DO ensure adequate fall protection devices are used at all times.

- DO face the ladder and use the three-point contact.

- DO keep metal ladders away from electrical sources.

- Do not use defective equipment or components.

- Do not use a harness or lanyard that has arrested a fallen worker.

- Do not attach two lanyards together to make them longer.