Safe Work Practice Skid Steer

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Awkward/sustained postures - bend, reach, lift |  |
| Forceful exertions - lifting |  |
| Repetitive movements |  |
| Vibration |  |
| Compression |  |
| Sharp points/edges - sharp materials |  |
| Pinch points - bin lids |  |
| Materials falling - bin lids |  |
| Surfaces causing falls - icy, snow in winter |  |
| Moving machinery |  |
| Chemicals |  |
| Biological pathogens - contact with waste |  |
| Electrical |  |
| Extreme heat/cold |  |
| Noise |  |
| Combustibles/flammables |  |
| Risk of falling |  |
| Other |  |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |

What are some good general safe work practices?

- Always Inspect area for hazards.

- Complete pre-operation inspection of Skid Steer.

- ONLY certified operators must run skid steer in hazardous situations.

- Understand the written instructions, rules, and regulations.

- Wear safety glasses, hearing protections, CSA approved footwear, and hard hat.

- KNOW your load capacities for forks & bucket & NEVER exceed them.

- Know the location of all workers & public at all times.

- Always know where overhead power lines are.

- No riders allowed.

- Always operate in a well-ventilated area.

- Always carry heavy end uphill.

- Always wear a seat belt.

- ONLY operate machine with lift bar down.

- Keep away from pinch points at all times.

- ONLY qualified personnel will “boost” skid steers.

- ALWAYS install lift arm stop when working on machinery while lift arms are up.

- Place jack stands at rear before lifting operator cab.

- ALWAYS use 3 points of contact while entering & exiting the skid steer.

- Follow safe practices & procedures for fueling.

- Before leaving skid steer, ensure lift arms are lowered, parking brake is on, and engine is stopped.

- ALWAYS double check that attachments are installed properly.

- Drive skid steer backwards onto trailers and secure according to manufacturer’s instruction.

- Always travel up & down slopes