Safe Work Practice Step Ladders

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Awkward/sustained postures - bend, reach, lift |  |
| Forceful exertions - lifting |  |
| Repetitive movements |  |
| Vibration |  |
| Compression |  |
| Sharp points/edges - sharp materials |  |
| Pinch points - bin lids |  |
| Materials falling - bin lids |  |
| Surfaces causing falls - icy, snow in winter |  |
| Moving machinery |  |
| Chemicals |  |
| Biological pathogens - contact with waste |  |
| Electrical |  |
| Extreme heat/cold |  |
| Noise |  |
| Combustibles/flammables |  |
| Risk of falling |  |
| Other |  |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |

Potential Health and Safety Hazards

- Ergonomic hazards; force and posture.

- Machine hazards; pinch points.

- Energy hazards; gravity and kinetic.

- Work practice hazards; following established safe work practices and procedures, and general housekeeping practices.

Safe Work Practices

- Read and follow the manufacturer’s instructions and warning labels.

- Wear appropriate personal protective equipment such as safety footwear (with heels and slip-resistant soles) and safety glasses.

- Know how to properly inspect and safely erect a portable ladder.

- Commercially manufactured portable ladders must be CSA approved.

- Only one person is permitted to be on a ladder at any time.

- Ensure that all electrical equipment that will be used during ladder work is in good working condition and is properly grounded.

- Ensure the work area is clear of debris.

- Ensure there is adequate lighting in the work area.

- Only use a stepladder for light duty tasks of short duration (less than 30 minutes) where your centre of gravity is maintained between the side rails.

- Do not use a ladder if you feel drowsy or unwell.

- Do not overreach while on a ladder. Move a stepladder when needed.

- Do not stand, climb, or sit on the top of a stepladder.

- Do not "shift" or "walk" a stepladder when standing on it.

- Do not use a stepladder as a brace or support for a work platform or plank. Do not climb a stepladder that is leaning against a wall. Use a straight ladder instead.

- Do not use a stepladder on a slippery surface.

- Do not use a stepladder on soft ground; the legs could sink into the ground.

- Do not place a stepladder on a box, unstable base or on a scaffold to gain additional height.

- Do not climb the back of a stepladder.

- Do not carry objects in your hands while climbing a ladder. Hoist materials or use a tool belt.

- Do not paint a wooden ladder.

- Do not try to straighten or attempt to use a bent or bowed ladder.

- Do not straddle the space between a ladder and another object/surface.

- Do not push or pull a stepladder sideways. Repeated sideways movement can make a stepladder wobbly.

- Do not use a ladder in a passageway, doorway, driveway, or other location where a person or vehicle could hit it. Set up suitable barriers or lock door.