Safe Work Practice TABLE SAW USE

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Awkward/sustained postures - bend, reach, lift |  |
| Forceful exertions - lifting |  |
| Repetitive movements |  |
| Vibration |  |
| Compression |  |
| Sharp points/edges - sharp materials |  |
| Pinch points - bin lids |  |
| Materials falling - bin lids |  |
| Surfaces causing falls - icy, snow in winter |  |
| Moving machinery |  |
| Chemicals |  |
| Biological pathogens - contact with waste |  |
| Electrical |  |
| Extreme heat/cold |  |
| Noise |  |
| Combustibles/flammables |  |
| Risk of falling |  |
| Other |  |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |

Health and Safety Hazards

- Chemical hazards: dust (harmful or nuisance dust, refer to MSDS).

- Physical hazards; noise.

- Machine hazards; moving parts/blades, flying objects (kickback), absence of guards and pinch points.

- Energy hazards; electricity and mechanical (moving parts).

- Work practice hazards; following established safe work practices and procedures, and general housekeeping practices.

Safe Work Practices

- Read and follow the manufacturer’s instructions and warning labels.

- Know how to safely operate the table saw and stop it quickly.

- Wear appropriate personal protective equipment such as safety footwear, safety glasses, full face shield, hearing protection and respiratory protection.

- When unplugging equipment pull on the plug, not on the cord.

- Keep power cords away from heat, water, and oil.

- Only use extension cords that are in good condition with proper grounding.

- Unplug any broken or unsafe equipment, attach a warning tag, take it out-of-service, and advise your supervisor.

- Ensure there is adequate lighting in the work area.

- Keep your hands/fingers away from the cutting area and moving blade.

- When the stock is less than 6 inches wide use a push stick to feed the stock.

- Ensure a push stick is close by prior to starting the table saw.

- Always use a rip fence or miter gauge/cut off guide to cut material.

- Ensure the blade guard; a splitter and an anti-kickback device are in place.

- Only cut material that is intended to be cut on a table saw (i.e., hardwood, softwood, plywood, OSB, other wood products, plastics, etc.).

- Do not force the material into the blade; use only a little pressure.

- Do not let a worker on the out-feed side of the table saw pull the material or squeeze the split pieces of material together.

- Do not over-reach when cutting material; keep proper footing and balance.

- Do not attempt to remove the material from the saw or pull it back while the blade is in motion.

- Do not use the saw table to stack material as you cut it.

- Do not use the saw to cross-cut long stock.

- Do not freehand cut any material on a table saw.

- Do not wear long sleeves, loose fitting clothing or jewelry while operating a table saw.

- Do not use a saw that vibrates or appears unsafe in any way.

- Do not make adjustments while the blade is in motion.

- Do not reach over/behind a moving saw blade to push stock through.

- Do not use the fence as a cut off gauge when cross cutting material.

- Do not attempt to pick up any cut off material while the blade is in motion.

- Never rip a piece of stock that is twisted, crooked, warped or does not have a straight edge to guide along the fence.