Safe Job Procedures

| Name of Job: Air Brake Adjustment | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Other workers and equipment | Machine malfunction | Repetitive motion strains |  |
| --- | --- | --- | --- |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Chocks/blocks | Safety vest | Hand protection |
| --- | --- | --- | --- |
| Hard hat | Safety glasses |  |  |

What is a proper safe job procedure to follow?

- Personal Protective Equipment should be worn at all times

- If unit has auto slack adjuster, do not try to adjust. See supervisor if travel is more than 2”.

- Park on level ground, out of the way of any traffic

- Lock out the ignition

- Make sure PSI will maintain 115 psi for duration of the adjustment

- Block/Chock wheels

- Release the brakes

- Check the brake lining thickness (min 3/8”)

- Check for air leaks and repair

- Check for damaged brake pot or slack adjuster, worn lining or leaking wheel seals

- Ensure lock sleeve is operational

- Ensure push rod travel is between ¾”-1 ½” and within ¼” of each other

- Ensure push rod angle is between 85 and 90 degrees (90 degrees being the best angle)

- Report any of the above problems to your supervisor

- Ensure brakes do not drag

- With psi at a minimum of 115, make on full brake application

- PSI should not drop more than 12 psi. If it does, recheck adjustment.