Safe Job Procedure for Bench Grinders

Pre Set-Up and Use of Grinder

- Ensure lighting is adequate.

- Do not wear loose clothing.

- Make sure there are no flammables around grinder

- Make sure the stone guard is set at proper angle to protect your eyes.

- Make sure the stone is no more than 1.55 mm to inside edge of rest.

- Make sure you’re wearing appropriate PPE.

- Make sure grinding stone is in good condition.

To replace a damaged grinding stone

- Review and follow specific manufacturer’s instructions.

- Unplug grinder from wall.

- Make sure rotation has stopped, push and hold lock pin to hold stone.

- Use appropriate wrench to loosen and remove arbour lock bolt.

- Replace stone with proper grade and size.

- Make sure nut is faced the right way depending on the application stone.

- Screw on the nut and tighten it firmly with the matching wrench.

- Remove finger from lock pin.

- Plug back into wall

Operation Procedure

- Start the grinder by pressing the switch to on.

- Firmly push the work piece down on the rest and slowly apply it to the stone.

- Be prepared to have sparks.

- While grinding, do not lift work piece off the rest. It will cause the work piece to catch the stone and possibly cause injury.

- Try and keep the work piece 90 degrees to the grinding stone or it might catch and pull it from your hands.

- Remove work piece from rest first pulling it away from stone.

- Turn off Grinder and do not leave grinder till it stops rotating.

- Keep the work area clean for other people to use it.

Additional Considerations

- Grinding produces heat so gloves are recommended, however, gloves should be snug fitted to avoid catching in moving equipment.

- Always make sure the saw is unplugged before changing the stone.

- Consult the supervisor if unsure of how to use.