Safe Job Procedures

| Name of Job: Changing Oxygen/Acetylene Tanks | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Explosion | Burns | Muscle strain |  |
| --- | --- | --- | --- |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Gloves | Face shield | Hard hat |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Close valves on tanks

- Press torch handle to disperse oxygen and acetylene

- Disconnect hoses

- Remove empty cylinders

- Replace with full cylinders

- Connect holding straps

- Reconnect hoses

- Open valves slowly

- Check for leaks with soap solution

- Ensure pressure gauges are set at the proper pressure

- Ensure oxygen and acetylene flow freely when turned on