Safe Job Procedures

| Name of Job: Changing a Propane Cylinder | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Explosion | Burns | Muscle strain |  |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Gloves | Face shield |  |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Check for date of manufacture or the last valid inspection date (10 years or less).

- Visually inspect the cylinder for cuts, gouges, dents and rusting.

- Test for leakage which may render cylinder defective.  Use soap or leak detector (service valve, valve, sight gauge, relief valve, vent valve).

- Check that the relief valve fitting is approximately 90O from the locating holes and secure.

- Check service valve for defects (handle broken, spindle bent, defective or missing “O” ring or back up ring.

- Fill valve cover in place (if applicable).

- Check to ensure valve is closed before connecting to coupler.

- Open flow control valve slowly (check for leakage).

- Tag defective cylinder, noting defect and report to a supervisor.