Safe Job Procedures

| Name of Job: Concrete Pipe Handling/Installation | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Pinch points | Potential injury or death | Muscle strain |  |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Hard Hat | Hand protection | Eye protection |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Use a machine for the lift that is rated for the proper weight

- Attach chain to pipe to be lifted by a competent rigger

- Have a spotter signal and guide the operator carefully and properly align the pipe

- Use wooden blocking for pipe

- Carefully clean all dirt and foreign substances from the jointing surface of the bell or groove end of pipe

- Lubricate bell jointing liberally with approved lube, using a brush, sponge or gloves

- Clean spigot or tongue end of pipe

- Lubricate gasket on spigot or tongue end

- Fit gasket carefully. Use a smooth, round object (i.e. screwdriver) inserted between gasket and spigot, and run it along the circumference several times

- Align bell and spigot to be jointed. Ensure gasket is in contact around entire circumference before homing the joint.