Safe Job Procedures

| Name of Job: Equipment Daily Maintenance | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Equipment failure | Collision | Pinch points |  |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Eye protection | Hand protection |  |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Walk around machine to check for tire damage, vandalism, any loose parts, or any other damage

- Check engine for:

  - Any frayed or damaged belts

  - Any damaged or leaking hoses

  - Engine mounting bolts are in place, tight and not damaged

- Check all fluid levels (some fluids may require the engine to be on, check manufacturers recommendations)

- Get into vehicle using 3 point contact method. Check for fire extinguisher and turn on engine

- Check all gauges:

  - Oil pressure is normal

  - Temperature is normal

  - Hydraulic pressure is normal (if gauge is applicable)

- Put machine in reverse to ensure back up alarm is working

- Do another walk around to check all lights

- Check brakes are working when pulling away to drive

- Proceed to job