Safe Job Procedure

| Name of Job: Fall Arrest Systems | Development Date: March 11th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Slips ,trips, falls | Overhead hazards | Falls From Heights | Risk of pain or injury from working in awkward positions, performing repetitive tasks, or lifting. |
| --- | --- | --- | --- |
|  |  |  | Use of various machinery and tools. |

Personal Protective Equipment (PPE) and Devices Recommended

| Hearing Protection | Eye Protection | Foot protection | Fall Arrest Equipment |
| --- | --- | --- | --- |
| Respirator Equipment | Skin Protection | Hearing Protection | Travel Restraint system. |

What is a Safe Job Procedure to follow?

- The supervisor is responsible for ensuring that all workers are trained & understand this procedure

- The supervisor shall ensure that all employees under their direction use a complete fall arrest system when a hazard of falling 3.0 meters or more exists

- All employees who require a fall arrest system shall be instructed in the inspection, donning and use of all

- components before the worker uses the system

- The employee is responsible for ensuring that a complete fall arrest system is used where there is a hazard of falling 3.0 meters or more & that this procedure is followed as directed by the supervisor

- Fall arrest equipment must be used by trained personnel only, & all equipment should be inspected by the

- the user before each use

- The fall arrest system consists of an approved full body harness and an approved lanyard

- All fall arrest system components shall be CSA approved and readily identified

- The length of the lanyard or the location of the anchor shall be so arranged that the worker can fall no farther than 1.2 meters. A retractable harness- mounted lanyard is the most acceptable method

- All fall arrest components shall be stored properly and kept in good condition

- Safety belts, harnesses, lanyards and lifelines shall not be knotted and shall not be allowed to become knotted or damaged

- Never wrap lanyards around sharp or rough anchor points

- Fall arrest equipment must not be altered in anyway

- Any equipment subjected to a fall must not be used again

- All synthetic materials must be protected from slag, hot sparks, open flames or other heat sources

- Maximum working load is 310 pounds, unless otherwise labelled

- The anchor or tie off point must be capable of supporting 5,000 pounds per worker