Safe Job Procedures

| Name of Job: Fire on the Work Site | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Inhalation of smoke | Inhalation of chemicals/toxins | Inhalation of carbon monoxide | Burns |
| --- | --- | --- | --- |
| Property damage |  |  |  |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Eye protection | Hand protection | Communication device |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Clear the area of any unauthorized personnel

- Determine the size of the fire:

  - Small – attempt to extinguish small fires only if no back up support and you are knowledgeable in fire fighting

  - Large: Fire Department should be called to put out large fire

- Notify Supervisor

- Remove one or more of the four elements of fire (oxygen, fuel, heat and chemical chain reaction) to extinguish

- Determine class of fire so proper extinguisher is selected:

  - Class A – occur in combustible materials (paper, wood, straw, cloth)

  - Class B – occur over the surface of flammable liquids (gasoline, oil, grease)

  - Class C – occur in energized electrical equipment

  - Class D – occur in certain combustible metals such as magnesium, titanium, potassium or sodium.

- Remember PASS when using a fire extinguisher

  - Pull the pin

  - Aim the extinguisher

  - Squeeze the trigger

  - Sweep at the base of the flames

- If attempt successful, all procedures may be ended. If NOT successful, provide the following information:

  - Exact location of fire

  - Type of fire

  - Whether medical assistance is required.