Safe Job Procedures

| Name of Job: Installation/Removal of Shoring | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Asphyxiation | Loose grounding | Muscle strain | Fall injuries |
| --- | --- | --- | --- |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Eye protection | Hand protection | Hard hat |
| --- | --- | --- | --- |
| Visibility vest | Hearing protection |  |  |

What is a proper safe job procedure to follow?

- While shoring is in progress the bucket of the excavator should be placed in the trench directly in front of shoring being installed

- It is essential that the struts or jacks be installed from the top down. The top strut/jack shall be placed 0.5 meters below the surface, and the second strut/jack placed according to shoring table

- If plywood is used, the jacks/struts must never be installed directly on the plywood. The jacks/struts must be placed on the uprights that support the plywood

- Once a minimum of 2 struts/jacks are placed on each set of uprights, the installation of the bottom jacks can be done

- This method allows the worker to be protected by the bucket of the excavator and the already installed shoring

- When removing shoring use the above method in the reverse order

- The bottom struts/jacks are removed first.