Safe Job Procedures

| Name of Job: Oil Changing | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Pinch points | Burns | rolling | Injury by machine |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Hand protection | Eye protection |  |
| --- | --- | --- | --- |

What is a proper safe job procedure to follow?

- Park vehicle, turn off the engine, set the brake, utilize lock out procedure and place wheel chocks.

- Place oil pail under the oil pan

- Remove plug and let drain. Take a clean sample of oil

- Loosen oil filter and remove. Dump contents of the filter in to the pail

- Clean and check threaded insert and filter base

- Lubricate sealing ring with clean oil, spin the filter until sealing ring contacts base then turn it once more

- Clean oil plug and replace it in the oil pan

- Open the hood, remove oil fill cap, place funnel in hole and fill with proper amount of oil, as per manufacturer specifications, then remove the funnel and replace the cap

- Check oil dipstick and ensure it is at the proper level, if not add more, start engine and let run for several minutes

- Turn engine off, look under the hood and under vehicle or unit to be sure there are no leaks