Safe Job Procedure for Table Saw Use

Pre-operation inspection and set-up

- Wear appropriate personal protective equipment.

- Disconnect the power cord from the electrical outlet before inspecting, adjusting, cleaning or repairing the saw.

- Check the work area for any debris or trip hazards; clean the area.

- Ensure the blade is sharp (dull blades bind and overheat).

- Ensure the blade is the correct type for the material and operation.

- Ensure the blade is properly seated, tightened and aligned.

- Ensure the blade is in good condition; no nicks, cracks or missing teeth.

- Check the blade for proper blade rotation.

- Ensure the throat plate is the correct type and it is properly installed.

- Check the stock for any defects such as warps or knots and foreign objects such as nails, staples or screws; remove all metal objects.

- Ensure there is enough room to maneuver the stock from front to back and to the side of the table saw.

- Ensure the guard and safety devices are in place and are working properly.

- If the stock is large or heavy get help, use auxiliary rollers and/or a table extension.

- Set the saw blade slightly higher than the stock; ¼ inch (6mm).

- Ensure the guide fence/miter gauge is locked into position.

- Ensure the guide fence is parallel with the blade.

- Ensure the stock is not touching the saw blade.

- Ensure the table is clear of all objects (e.g. tools, scraps of wood, etc).

- General Operation

- Power up the table saw.

- Allow the blade to come to full speed before starting to cut the stock.

- Stand to one side (left or right) while operating the saw. In case the material kicks back you do not want your body in line with the blade.

- Using the miter guide or push stick to push stock through the blade.

- Pass the stock completely through the blade then power down the saw.

- Wait for the blade to completely stop before removing the material.

- Remove any debris from the table top before cutting other pieces.

- Turn the saw blade down to the lowest position when finished.

- Ripping

- Hold the stock down on the table and firmly against the guide fence.

- Start the cut slowly.

- Hold the stock with both hands and push it along the fence into the saw blade. Use a push stick if the material is less than 6 inches wide.

- Continue to feed the stock through the blade until it is completely cut and past the anti-kickback device.

- Remove the larger outside piece and place it out of the way.

- Do not try to remove the cut off stock (piece adjacent to the fence) until the blade has completely stopped.

- Cross Cutting

- Hold the stock against the miter gauge and down on the table. Do not hold the piece that is being cut off.

- Start the cut slowly. Continue the feed until the material is cut.

- Shift the work piece sideways to move it away from the blade.

- Pull the miter gauge and work piece back to the start position.

- Do not try to remove the cut off stock until the blade has completely stopped.