Safe Job Procedures

| Name of Job: Welding | Development Date: March 30th, 2011 | Developed By: CSNS |
| --- | --- | --- |

Possible Hazards Present

| Burns | Electrical shock | Inhalation of gases | Flash |
| --- | --- | --- | --- |
|  |  |  | . |

Personal Protective Equipment (PPE) and Devices Recommended

| Steel toed boots | Eye protection | Leather gauntlet gloves | Welders mask |
| --- | --- | --- | --- |
| Welding curtain | Fire extinguisher | Respiratory protection | Local exhaust |

What is a proper safe job procedure to follow?

- Only qualified personnel may perform welding activities

- Always work with a spotter (fire watch) to ensure that a spark or fire is eliminated

- Ensure the work area is well ventilated or that provided ventilation hoods are in use

- Check the work area for combustible material and possible flammable vapors before starting work

- Check all cables and hoses to protect from slag, heat, fire and sparks. Check cables and hoses for damage before starting

- Before welding drums, tanks, etc. that have been in service, ensure that all precautions have been carried out and permits have been obtained, as required

- Ensure that the ground cable is applied to the frame of the object being welded

- Always wear the appropriate mask/goggles

- Ensure other workers are protected from arc flash burns by utilizing screens or advising people of the work being performed in the area