Safe Job Procedure for Confined Spaces

- Identifying Confined Space

- Determine if the space is considered a “confined space” as per the OH&S legislation (i.e. not designed or intended for human occupancy except for the purpose of performing work, has restricted access/egress and may become hazardous to a person entering it).

- Hazard Assessment

- A worker is not permitted to enter a confined space until a person with adequate knowledge, training and experience has completed a formal hazard assessment and the Confined Space Entry Permit.

- Where two or more confined spaces are of similar construction and present the same hazards, their hazard assessments may be recorded in a single Confined Space Hazard Assessment Form, however, they each require a completed and separate Confined Space Entry Permit.

- Air Monitoring

- There must be appropriate tests for harmful substances and oxygen deficiency before entry into a confined space, after an interruption in the work procedure and at appropriate interval (preferably done on a continuous basis)

- Tests must be performed by a worker who has been adequately trained.

- If a test indicates an unsafe condition, then the confined space must be ventilated or cleaned or both and periodically retested.

- Test results must be recorded on the entry permit form.

- Entry Permit

- A worker is not permitted to enter a confined space without a valid entry permit.

- The confined space entry permit is valid only for the specific confined space.

- The completed and signed entry permit must be posted at the entry portal to the confined space.

- Hot Work Permit

- If hot work is involved in the confined space, then a special hot work permit must be obtained from the respective maintenance supervisor.

- Zero LEL’s must be maintained at all times or other form of control method must be implemented.

- Attendant and Entrant Responsibilities

The “Attendant” is required to;

- Remain alert at all times outside and near the entrance.

- Monitor the safety of the entrant in the confined space.

- Control and monitor the lifeline of the entrant.

- Provide assistance to the entrant.

- Maintain continuous 2-way communication with the entrant.

- Initiate an emergency response when necessary. Perform non-entry emergency rescue if safe to do so.

The “Entrant” is required to;

- Remain alert at all times while working in the confined space.

- Be aware of all potential hazards in the confined space.

- Be aware of any symptoms or warning signs of exposure.

- Immediately report any concerns to the attendant.

- Immediately exit the confined space if conditions warrant it, an alarm sounds or if directed to do so.

- Communications

- The attendant and entrant must be in continuous communication with each other.

- The attendant must be provided with a communication device for summoning an adequate rescue response.

- There must be in place a backup form for all communications.

- Implementation of Hazard Controls

- Identified hazards must be properly controlled by implementing appropriate control methods.

- Workers must ensure their PPE is in good working condition and is suitable for the type of work to be performed.

- A full body harness with a lifeline must be worn where it does not pose a hazard.

- Refer to the Respiratory Protection Program if a respirator is required.

- Entry into a Confined Space

A worker is permitted to enter a confined space only where;

- The opening is sufficient to allow safe passage of a worker including PPE and rescue equipment;

- Mechanical and electrical equipment in the confined space is locked out;

- Pipes and other supply lines are blanked off;

- Measures have been taken to ensure that, where appropriate, the confined space is continuously ventilated;

- A liquid or free-flowing solid has been removed;

- Explosion-proof illumination is provided where appropriate; and

- Adequate barriers are erected to prohibit unauthorized entry.

- Emergency Rescue

- Rescue procedures must be established and reviewed prior to any worker entering a confined space.

- If rescue is required the attendant must immediately notify rescue personnel, the local fire department, emergency medical services and the respective maintenance supervisor.

- Rescue personnel, equipment and first aid supplies must be readily available for the duration of the entry.

- Under no circumstances shall the attendant enter the confined space to perform a rescue.