Safe Job Procedure Handling and Disposing of Sharps

(sharp objects that may have been in contact with blood and bodily fluids)

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Sharp points/edges | High |
| Biological pathogens | High |
| Other |  |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |
| - Gloves - Eye goggles - Laboratory coats/aprons | - WHMIS 2015 - CAN/CSA-Z316.6-14-Sharps Injury Protection - Requirements and Test Methods - Sharps Containers - National Standard of Canada/Canadian General Standards Board-43.125- 2016 (CAN/CGSB-43.125-2016). - TDG Regulation |

Note: Common signs and symptoms of a musculoskeletal injury (MSI) can include pain, burning, swelling, stiffness, numbness/tingling, and/or loss of movement or strength in a body part. Report these to your supervisor.

Employers must ensure that workers are trained and follow this safe work procedure

## Steps to complete this task safely:

- Ensure a sharps container is within reach.

- After using a sharp, hold the upper end pointed away from you, and ensure no one is between you and the sharps container.

- Discard all used syringes as a single unit into the sharps container.

Do not:

- Insert fingers into opening of sharps container

- Recap needles

- Separate needles and syringes

- Dispose of sharps in the garbage

If needle and syringe become separated, use forceps to dispose of needle.

- If the sharp is a scalpel, use forceps to remove the blade from the non-disposable scalpel handle without breaking the blade. Place the blade in the sharps container.

- When finished, remove gloves and contaminated clothing and dispose of them into a biohazardous waste container.

- Wash hands with soap and water, followed by a sanitizer.

- Before leaving the workspace, ensure all sharps are disposed of correctly and all spills are appropriately cleaned.