Safe Job Procedure Moving Furniture

| Department/Area: | Approved by: | Date Created: | Review/Revision Date: |
| --- | --- | --- | --- |
| insert text here |  |  |  |

| Potential Hazard | Risk level |
| --- | --- |
| Awkward/sustained postures - bulky, awkward | High |
| Forceful exertions - heavy loads | High |
| Repetitive movements | N/A |
| Vibration | N/A |
| Compression | N/A |
| Sharp points/edges | Medium |
| Pinch points - Fingers jamming | Low |
| Materials falling | N/A |
| Surfaces causing falls | N/A |
| Moving machinery | N/A |
| Chemicals | N/A |
| Biological pathogens | N/A |
| Electrical | N/A |
| Extreme heat/cold | N/A |
| Noise | N/A |
| Combustibles/flammables | N/A |
| Risk of falling | Low |
| Other | N/A |

| Risk control devices, personal protective equipment, and other safety considerations | Training/Reference info |
| --- | --- |
| - Gloves when there is rough material or sharp edges. - Two people are required to lift large, bulky, or heavy items over 20 kg, or if exertion to move an item would require more than moderate effort. | - Safe lifting orientation - Safe work procedure on manual materials handling |

Note: Common signs and symptoms of a musculoskeletal injury (MSI) can include pain, burning, swelling, stiffness, numbness/tingling, and/or loss of movement or strength in a body part. Report these to your supervisor.

Employers must ensure that workers are trained and follow this safe work procedure.

## Steps to complete this task safely:

- Assess the load. Do not attempt to lift or carry heavy or awkward loads by yourself. Use a lift aid (wheeler, cart, etc.) or get assistance.

- Wear gloves when lifting or moving sharp-edged or rough materials. Beware of any pinch points (such as desk drawers or table leg mechanisms).

- To slide or roll objects or furniture away from you, try to keep your back straight and weight shift with your whole body rather than using only your arms. Whenever possible, push objects or furniture rather than pulling them because this allows a safer back position.

- When moving furniture slightly or adjusting the furniture's position, sometimes it's effective to place your leg against the item and lean against it.

- If possible, ensure there is enough room to lift properly and travel by removing any obstructions.

- Proper lifting and carrying procedures are to:

- use a wide stance either side to side or front to back (lunge position). Get close to what you must lift.

- get a good grip before lifting.

- bend your knees and hips.

- try to keep your back straight, shoulders back, and elbows in.

- lift by pushing with your legs. If using a lunge position, push more through your front leg.

- don’t hold your breath.

- keep the load close to your body when lifting or carrying.

- turn by taking small steps with your feet. Do not twist your back when lifting, lowering, or carrying.

- When two or more people are lifting or carrying a load, the method for handling the object should be decided before lifting, and one person should determine the count to lift.