Safe Work Practice

Extension Cords

Extension cords can cause serious accidents if the following practices are not followed.

- All electrical extension cords must be designed for outdoor use, rated for 300 volts, and have a functioning insulated grounding conductor.

- All extension cords will be CSA-approved and inspected before use.

- Extension cords are to be protected against damage from sharp edges, movement of materials, and flame cutting.

- All frayed, cut, spliced, or defective extension cords are to be tagged and removed from service immediately until they can be replaced or repaired.

- All extension cords are to be placed in such a way that they will not be a tripping or falling hazard.

- All extension cords used in hazardous areas or in damp locations are to be protected by approved ground fault protection.

- Extension cords should be plugged into Class A ground fault circuit interrupters (GFCIs). When built-in GFCI receptacles are not available, protection can be attained with an in-line GFCI plugged directly into the supply receptacle.

- Use extension cords that are the correct length for what you need. DO NOT plug shorter cords together. This can cause a cord to overheat and create a fire hazard.