Safe Work Practices – Identifying Asbestos

As part of the ongoing commitment to provide a safe work environment, the following procedure has been established to address the existence of asbestos on job sites.

What is Asbestos?

Asbestos is a naturally occurring material once used widely in the construction industry. Its strength, ability to withstand high temperatures, and resistance to many chemicals made it useful in hundreds of applications.  However when asbestos is inhaled, it can be harmful and lead to the following diseases:

- asbestosis

- lung cancer

- mesothelioma (cancer of the lining of the chest and/or abdomen).

Where can it be found?

- Sprayed-On Fireproofing

- Pipe and Boiler Insulation

- Loose Fill Insulation

- Asbestos Cement Products

- Acoustical Plaster

- Acoustical Tiles

- Vinyl Asbestos

- Gaskets

- Roofing Felts

- Asphalt/Asbestos Limpet Spray

- Drywall Joint-Filling Compound

- Coatings and Mastics

Prior to Commencing Work

Supervisors:

- Prior to commencing work in any area, request a copy of the owner’s Asbestos Report.

- If there is asbestos in the work area and it needs to be removed to perform the work, request that the owner to remove it.

- Do not commence work until you have received a notice from the owner in writing that the asbestos has been removed and it is safe to commence or return to work.

- If there is asbestos in the work area and its presence does not impact the work, advise workers of location and what not to disturb.

Workers:

- In all projects, bear in mind the possible presence of asbestos.

- If you are working in an area known to contain asbestos, contact your supervisor to determine whether or not asbestos is present in the work area or adjacent areas in which you may be working.

- If there is any doubt about pipe or duct insulation, textured ceilings, vinyl asbestos floor tile, flooring sheet goods, wall cladding or underground piping, especially in older facilities, do not commence work and notify your supervisor.

Note:  To remove Asbestos a worker requires knowledge of the type of asbestos, knowledge of the proper choice and use of PPE and Respirators, understanding of containment procedures and knowledge of proper handling, storage and waste removal procedures.

DO NOT REMOVE OR DISTURB ASBESTOS CONTAINING MATERIAL.  IF YOU ARE INSTRUCTED TO DO SO, STOP WORK AND CONTACT YOUR SUPERVISOR.